MEDICAL MONITORING AND ADVANCED SENSOR DATA PROCESSING

TASK 01: DATA ACQUISITION BASICS

Emilio Borrelli, Victor Hug, Eva Pohlen 5-21-2025

# Tasks

Q2	Discuss the recording technique and lead the smartwatch yields compared to

using a standard ECG

A standard ECG is recorded using multiple electrodes, with their exact placements (referred to as “leads”) depending on the standard used. The most common lead is Einthoven II, where an electrode is placed on the subject’s right wrist and left foot. All leads yield different results, since the ECG amplitude, in simplified terms, is derived from 3D vectors projected onto a 2D plane. The different leads in this case are different angles from which the vectors are viewed. Multiple different leads are often measured simultaneously to account for that. The smartwatch uses the Einthoven I lead; one electrode placed on each of the subject’s wrist. This yields less pronounced amplitudes than the Einthoven II. The QRS complex is much flatter, with the Q entirely invisible and the R amplitude significantly reduced. The P- and T-waves are less pronounced and hard to separate from background noise. There is also more noise in the data than in a recording with professional equipment.

Q5	Perform a recording and plot 10 seconds of a PPG signal along with the

accelerometer signal. What are the signal names in the recording file and to which

measurement modality do they correspond to?

PPG Timestamp: Time (in Unix time, down to a thousandth of a second) of recording for the PPG value of each entry

PPG: Value measure by PPG sensor, measurement of blood pressure

ADXL Timestamp: Time (in Unix time, down to a thousandth of a second) of recording for the accelerometer values of each entry

X: X-axis (left/right) value of accelerometer

Y: Y-axis (depth/inward) value of accelerometer

Z: Z-axis (vertical/gravity) value of accelerometer

Q8	Make 2 plots (one above the other) of the ECG from the Biopac device and

the smartwatch. What do you notice about the position of the artefact you introduced on both

plots? Make 2 plots (one above the other) of the PPG from the Biopac device and the

smartwatch. What do you notice about the position of the artefact you introduced on both

plots?

The artefact appears with a slight delay in the recording from the watch compared to the one done with the Biopac device, in both the ECG and PPG data.

It also takes a different form. In the BioPac recordings, the artifact is visible as short period of strong positive and negative (in relation to the baseline) amplitudes appearing in rapid succession. In the smartwatch ECG the artifact appears as two extreme spikes in both directions, followed by a slow return to the baseline. In the smartwatch PPG the artifact appears as a sudden spike in blood pressure. However, after the spike the baseline blood pressure stays on the level of that spike, so it is possible that we failed to introduce a visible artifact entirely, and this is just a natural spike in blood pressure that happened to occur at the same time as our attempt to create noise in the data.

Q9 	Based on the 2 last questions, propose a way to synchronize the ECG and

PPG from the Biopac device with those from the smartwatch.

(Maximum one 10 lines paragraph)

Ideally, syncing the artifacts would sync the data. However, since the artefact was introduced manually, the delay we observed could be a result of uneven timing in disturbing the sensors. We suggest aligning the R-peaks in the ECG data, and the systole and diastole peaks or the PPG data.

Q11	Make 2 plots (one above the other) of the histograms computed from ECG

signals recorded with the Biopac device and the smartwatch. How do you interpret these

histograms?

Q12	Make 2 plots (one above the other) of the histograms computed from PPG

signals recorded with the Biopac device and the smartwatch. How do you interpret these

histograms?

# Literature

[1] 02 Monitoring fundamentals.pdf (Lecture Slides)